Dijiste:

Badesa et al. - 2019 - Simultaneous Scheduling of Multiple Frequency Serv.pdf

PDF

read the following paper and say read when you finish

ChatGPT dijo:

I’ve finished reading

Badesa et al. - 2019 - Simultan…

.

Read.

Fuentes

Dijiste:

show me equation 13 from the paper

ChatGPT dijo:

Equation (13) in the paper is the frequency nadir constraint (derived without load damping). It is:

(Hf0−RS TS4 Δfmax⁡) RG  ≥  (PL−RS)2 TG4 Δfmax⁡(f0​H​−4Δfmax​RS​TS​​)RG​≥4Δfmax​(PL​−RS​)2TG​​

This inequality ensures the frequency nadir requirement is satisfied after a generation outage.

Badesa et al. - 2019 - Simultan…

Fuentes

Dijiste:

PL​−RS​

can it happen that this term is negative?

ChatGPT dijo:

If you mean the term

PL−RSPL​−RS​

then mathematically yes — but physically (and in the paper’s operating assumptions) no.

✔ Mathematical view

Nothing in pure algebra prevents PL−RS<0PL​−RS​<0.
That would mean:

RS>PLRS​>PL​

i.e., enhanced frequency response (EFR) exceeds the size of the largest power loss.

✔ Physical / power-system interpretation

That would imply the system injects more fast response power than the lost generation, so instead of frequency falling, it would rise after the disturbance (over-compensation).

In real frequency-security design, response is scheduled to cover the largest loss, not exceed it significantly. Over-response would be undesirable and normally avoided by scheduling and control.

✔ What the paper assumes

The paper explicitly assumes the nadir occurs after TSTS​ if EFR is lower than the largest infeed, and states this is certainly the case for GB operation.

Badesa et al. - 2019 - Simultan…

So their modelling effectively assumes:

RS<PLRS​<PL​

✔ In Equation (13) specifically

Even if it were negative, the term appears squared:

(PL−RS)2(PL​−RS​)2

so the inequality itself still makes mathematical sense. But the physical derivation and interpretation rely on RS<PLRS​<PL​.

✅ Summary:

- Algebraically possible.

- Operationally unlikely and not intended in the model.

- The paper assumes PL−RS>0PL​−RS​>0.

If you want, I can explain what happens to the frequency trajectory when RS>PLRS​>PL​.

Fuentes

Dijiste:

what happens in the system if a smaller contigency thab Pl occurs

ChatGPT dijo:

Good question — this gets to the security philosophy behind using PLPL​.

In the paper, PLPL​ is the largest credible contingency (largest possible sudden power loss). The system is scheduled so that frequency limits are satisfied for that worst-case event.

Badesa et al. - 2019 - Simultan…

If a smaller contingency than PLPL​ occurs

Suppose the actual loss is:

ΔP<PLΔP<PL​

1️⃣ Frequency drop will be smaller

All key frequency effects scale with the disturbance size:

- RoCoF (initial slope) ↓

RoCoF∝power lossinertiaRoCoF∝inertiapower loss​

- Frequency nadir will be higher (less drop)

- Steady-state deviation will be smaller

So the system response is more secure than required.

2️⃣ Frequency response becomes “more than enough”

The system has already scheduled inertia, EFR, and PFR to handle PLPL​.

If the disturbance is smaller:

- Fast response injects power sized for a bigger loss

- Frequency decline stops earlier

- Recovery is faster

In practice, control systems (governors, storage controllers) regulate continuously, so they naturally provide less response if the frequency deviation is smaller. They don’t blindly inject the full scheduled MW unless needed.

3️⃣ Possible small overshoot during recovery

Because response is designed for a worst case, you may see:

- Slight over-recovery (frequency briefly above nominal)

- Then normal governor control settles frequency back

This is normal and acceptable.

4️⃣ Operational interpretation

Designing for PLPL​ ensures:

✅ All smaller contingencies are automatically secure
✅ No need to reschedule response in real time
✅ Conservative but reliable operation

This is standard N-1 security practice in power systems.

Intuition

Think of it like designing brakes for emergency stopping at 120 km/h.
If you’re only going 60 km/h, the same brakes just stop you faster — safely.

Summary

If a contingency smaller than PLPL​ happens:

- Frequency drop is smaller

- Security limits are easily satisfied

- Response is more than sufficient

- System stabilizes faster (maybe slight overshoot)

The system is over-protected by design.

If you'd like, I can explain how the swing equation response scales with contingency size step-by-step.

Fuentes

Dijiste:

I want to model another kind of contingency where there is another PL but related to the loose of power infeed from a converter. I plan to use the same constraints, but in many cases, the loss of power infeed from converters can be smaller than the generators. Does it make sense to define another Pl variable for the loss of a converter, and use the same equations even though the loss of power can be smaller than the procured fast reserve?

ChatGPT dijo:

Yes — it can make sense to introduce a second contingency size variable for a converter loss, but you’ll want to tweak how you use the constraints, because the assumptions behind (13) (and the “nadir occurs in [TS,TG)[TS​,TG​)” logic) can break when the loss is small relative to fast response.

Here’s a clean way to think about it.

1) Two credible contingencies is a standard extension

If you have two independent “largest credible losses”:

- PLsynPLsyn​: largest synchronous infeed (generator trip)

- PLconvPLconv​: largest converter infeed (HVDC link, grid-forming/inverting plant block, etc.)

then frequency security should hold for both. The usual way is:

enforce the frequency constraints for each contingency, and take the binding one.

So you’d duplicate the RoCoF, nadir, and quasi–steady-state constraints with PLPL​ replaced by PLsynPLsyn​ and PLconvPLconv​.

That part is conceptually sound.

2) The catch: small converter losses can invalidate the “shape” assumed by Eq. (13)

Equation (13) is derived assuming (among other things):

- the nadir occurs before TGTG​, and

- (in their GB context) typically after TSTS​ because RS<PLRS​<PL​.

Badesa et al. - 2019 - Simultan…

If you have cases where

PLconv≤RSPLconv​≤RS​

(i.e., the converter loss is smaller than the procured fast reserve / EFR capability), then using (13) as-is can become:

- over-conservative, and/or

- not the right condition, because frequency might not even decline past TSTS​, and could start recovering earlier (or never “nadir” in the assumed interval).

Mathematically, (13) still “works” because (PL−RS)2(PL​−RS​)2 is nonnegative, but physically the derivation is no longer aligned with the real trajectory.

3) What’s the right modeling approach?

Option A (robust and simple): keep one PLPL​ = worst credible loss

If the system must be secure for either event, the simplest is:

PL=max⁡(PLsyn, PLconv)PL​=max(PLsyn​,PLconv​)

and just run the single set of constraints.

This is conservative but often acceptable, and it avoids all corner cases.

Option B (recommended if you really want two categories): enforce constraints for each contingency, but cap EFR in the converter-loss constraints

If your issue is “converter loss is smaller than procured fast reserve”, then for the converter-loss constraints you can define an effective EFR that can actually be activated by that event. Two common ways:

B1) Event-limited activation (good practical hack):

RSeff,conv≤PLconvRSeff,conv​≤PLconv​

and then use RSeff,convRSeff,conv​ (instead of RSRS​) inside the converter-loss nadir and q-s-s constraints.

Interpretation: for a small disturbance, you don’t need / won’t deploy more fast response than the lost power.

You can still keep the original RSRS​ for the generator-loss constraints.

B2) Piecewise “min” (more accurate, MILP-able):

RSeff,conv=min⁡(RS, PLconv)RSeff,conv​=min(RS​,PLconv​)

This is easy to linearize with standard MILP constructs (additional binary + big-M), and it makes the constraints behave correctly when PLconv<RSPLconv​<RS​.

Then in the converter-loss version of (13)/(19)/(9) you replace RSRS​ by RSeff,convRSeff,conv​ and also replace (PL−RS)(PL​−RS​) by (PLconv−RSeff,conv)(PLconv​−RSeff,conv​).

This keeps the math consistent with the assumption that the “residual imbalance after fast response” is nonnegative.

Option C (most correct): derive nadir constraints for both time regions

If you want full fidelity, you can derive two nadir conditions:

- one for nadir in t∈[0,TS)t∈[0,TS​) (EFR still ramping),

- one for nadir in t∈[TS,TG)t∈[TS​,TG​) (their case),

and enforce the appropriate one depending on whether PLPL​ is above/below the available fast response. This becomes a piecewise model.

This is more work, but it’s the cleanest theoretically.

4) Does it “make sense” to use the same equations?

- RoCoF constraint: yes, it scales cleanly with loss size.

- Quasi-steady-state constraint: yes, but if you allow over-response you may get sign flips; practical implementations usually avoid scheduling response > loss for that event.

- Nadir constraint (Eq. 13 / 19): use with care. If PLconv<RSPLconv​<RS​, the original derivation assumptions don’t match; you should use an effective/capped fast response (Option B) or a piecewise nadir model (Option C).

Practical recommendation

If you want something implementable that behaves well:

- Introduce PLsynPLsyn​ and PLconvPLconv​.

- Duplicate the frequency constraints for each contingency.

- For converter-loss constraints, introduce

RSeff,conv=min⁡(RS, PLconv)RSeff,conv​=min(RS​,PLconv​)

and use RSeff,convRSeff,conv​ in place of RSRS​ in the converter-loss nadir/q-s-s constraints.

That will prevent weird cases where the converter-loss constraints become nonsensical because the event is smaller than the procured fast response.

If you share how you’re modeling EFR (as headroom, as a symmetric step, as “charge-to-discharge swing” like the paper), I can write the exact MILP constraints for RSeff,conv=min⁡(⋅)RSeff,conv​=min(⋅) in your notation.

Fuentes

Dijiste:

If I have an equation where pl is less than rs for the converters, wouldn't adding it simply mean that the equation is becoming redundant, and would it become binding where the pl of the converters were greater than the pl of the generators?

ChatGPT dijo:

Mostly yes — with one important nuance.

If PLconv<RSPLconv​<RS​, does adding the converter constraint become redundant?

If you literally duplicate the paper’s constraints and just substitute PL→PLconvPL​→PLconv​:

- RoCoF: it will be easier to satisfy (smaller loss), so it’s typically redundant.

- Quasi–steady-state (their eq. 9): with PLconv−RS−RGPLconv​−RS​−RG​ it’s also typically redundant (it becomes “more than enough response”).

- Nadir (eq. 13 / 19): it will often be redundant too because the “required” response decreases as PLPL​ decreases, and the RHS uses (PL−RS)2(PL​−RS​)2, so if PLconvPLconv​ is small the RHS tends to shrink.

So in many normal cases: yes, the converter-loss version won’t bind if PLconvPLconv​ is small relative to the already-procured RSRS​.

Will it become binding only if PLconv>PLgenPLconv​>PLgen​?

If you define a separate contingency and enforce constraints for both, then the binding set is basically the one with the larger “effective stress” on the system.

Very often that aligns with the larger contingency size, so:

- if PLgen≥PLconvPLgen​≥PLconv​, generator-loss constraints dominate,

- if PLconv>PLgenPLconv​>PLgen​, the converter-loss constraints can dominate.

So your intuition is right as a first-order rule.

The nuance: “effective stress” isn’t only about the raw PLPL​

Even if PLconvPLconv​ is smaller, it could still bind if the assumed response/inertia differs by event. Examples:

- Converter trip might be correlated with low inertia periods (high RES), so the same PLPL​ is “harder”.

- Converter events might have different response availability (e.g., some EFR is provided by the same converter/BESS behind it and becomes unavailable).

- Different protection / control may cause different deadbands / delays (not in the paper’s simplification, but in reality).

If you model both contingencies using the same H,RS,RGH,RS​,RG​ availability, then yes: the larger PLPL​ case tends to dominate.

One more detail: when PLconv<RSPLconv​<RS​, the paper’s nadir derivation assumptions may not hold

Even if the constraint becomes redundant, it might be “redundant for the wrong reason,” because eq. (13) was derived assuming a particular nadir timing region (their discussion around t∗t∗ and TSTS​).

Badesa et al. - 2019 - Simultan…

So you can include it and it won’t hurt feasibility, but it may not be the right physics for small events.

Best practice (simple and safe)

If your goal is: “ensure security for both generator loss and converter loss,” and converter losses are usually smaller:

- You can include both constraint sets; the converter set will usually be slack.

- It will become binding when converter losses are larger or when you model event-specific availability.

If you want it to be physically consistent when PLconv<RSPLconv​<RS​, the clean trick is to use an event-effective fast response:

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

so the converter constraint doesn’t rely on “over-response” regimes.

If you tell me whether, in your model, RSRS​ is the scheduled headroom or an automatic proportional response, I can suggest the tightest (and still linear) formulation.

Fuentes

Dijiste:

RSeff,conv​

what is this?

ChatGPT dijo:

It’s just a modeling helper variable — an effective fast response for the converter contingency.

Definition

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

What it represents physically

It means:

👉 For the converter-loss event, you only count the fast response that is actually needed (or can be activated).

If the converter loss is small, you don’t assume the system deploys more fast response than the lost power.

Why this is useful

In your situation:

- You schedule total fast response capacity = RSRS​

- But the converter loss may be smaller than that

Example:

| Quantity | Value |
| --- | --- |
| Scheduled EFR capacity RSRS​ | 600 MW |
| Converter loss PLconvPLconv​ | 200 MW |

Physically:

- You won’t inject 600 MW for a 200 MW loss

- Only ~200 MW of response is actually relevant

So:

RSeff,conv=200 MWRSeff,conv​=200 MW

What problem it fixes

The paper’s nadir and steady-state constraints assume response is sized relative to the disturbance.

If you use the full RSRS​ when RS>PLconvRS​>PLconv​:

- The math may enter a regime not covered by the derivation

- It can imply unrealistic “over-response”

- Constraints may look satisfied for the wrong reason

Using the effective response keeps the physics consistent.

How it is used in equations

For generator loss:

RS(unchanged)RS​(unchanged)

For converter loss:

RS→RSeff,convRS​→RSeff,conv​

So the same constraint structure applies, but with event-specific usable response.

Intuition

Think of RSRS​ as:

🧰 Tools available in the toolbox

and RSeff,convRSeff,conv​ as:

🔧 Tools actually needed for this specific repair

You don’t use a sledgehammer to push in a thumbtack.

Important: this does NOT change procurement

You still procure RSRS​ to cover the largest credible event.

This only affects:
✔ how each contingency is evaluated
✔ how constraints interpret response for that event

Summary

RSeff,convRSeff,conv​ is:

👉 the fast response that is effectively relevant for the converter-loss event
👉 equal to the smaller of available response and event size
👉 a modeling device to keep nadir physics consistent when events are small

If you want, I can show you the exact MILP linear constraints to implement

RSeff,conv=min⁡(RS,PLconv)RSeff,conv​=min(RS​,PLconv​)

in your optimisation.

Dijiste:

yeah, do it

ChatGPT dijo:

Here are two standard MILP formulations you can drop into your model to enforce

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

I’ll use:

- RS≥0RS​≥0 = procured/available fast response (EFR)

- PLconv≥0PLconv​≥0 = converter contingency size

- RSc≥0RSc​≥0 = effective fast response for the converter event (what you’ll use inside the converter-loss frequency constraints)

- MM = a sufficiently large constant (upper bound on RSRS​ and PLconvPLconv​)

- z∈{0,1}z∈{0,1}

Option 1 (simple, common): “min” with one binary + big-M

Add these constraints:

RSc≤RSRSc​≤RS​RSc≤PLconvRSc​≤PLconv​RSc≥RS−M(1−z)RSc​≥RS​−M(1−z)RSc≥PLconv−MzRSc​≥PLconv​−Mzz∈{0,1}z∈{0,1}

How it works

- If z=1z=1: the last inequality becomes RSc≥PLconv−MRSc​≥PLconv​−M (loose), and the third becomes RSc≥RSRSc​≥RS​. Together with RSc≤RSRSc​≤RS​, you get RSc=RSRSc​=RS​. This corresponds to the case RS≤PLconvRS​≤PLconv​.

- If z=0z=0: the third becomes RSc≥RS−MRSc​≥RS​−M (loose), and the fourth becomes RSc≥PLconvRSc​≥PLconv​. Together with RSc≤PLconvRSc​≤PLconv​, you get RSc=PLconvRSc​=PLconv​. This corresponds to the case PLconv≤RSPLconv​≤RS​.

Pick MM tight (important for performance):

M≥max⁡{RSmax⁡,  (PLconv)max⁡}M≥max{RSmax​,(PLconv​)max}

If you already have bounds (e.g. RS≤RS,max⁡RS​≤RS,max​, PLconv≤PL,max⁡convPLconv​≤PL,maxconv​), set M=max⁡(RS,max⁡,PL,max⁡conv)M=max(RS,max​,PL,maxconv​).

Option 2 (even tighter if you already have a binary describing the active branch)

If you already have a binary that indicates whether the converter loss is the limiting one (or you want to reuse an existing “event mode” binary), you can reuse it as zz above and avoid adding new binaries. Otherwise Option 1 is best.

How to use it in your frequency constraints

For the converter-loss constraints, replace every place where the paper uses RSRS​ in the disturbance balance by RScRSc​, and replace (PL−RS)(PL​−RS​) with (PLconv−RSc)(PLconv​−RSc​).

Examples:

Quasi–steady-state (paper eq. 9 form)

Original:

∣Δfss∣=PL−RS−RGD⋅PD≤Δfssmax⁡∣Δfss​∣=D⋅PD​PL​−RS​−RG​​≤Δfssmax​

Converter version:

PLconv−RSc−RGD⋅PD≤Δfssmax⁡D⋅PD​PLconv​−RSc​−RG​​≤Δfssmax​

(and typically also enforce the symmetric bound if you model both directions)

Nadir constraint (paper eq. 13 / 19 style)

Wherever you have (PL−RS)(PL​−RS​), use (PLconv−RSc)(PLconv​−RSc​). That prevents the “over-response” regime from creating weird behavior.

Practical note

This formulation doesn’t force the system to actually deploy only RScRSc​ in reality — it just makes your security constraints consistent with the fact that a small event can’t “need” more response than the lost infeed.

If your controller is droop-based, this is also aligned with physics: smaller ΔfΔf ⇒ less response.

If you paste your exact converter-loss version of the nadir constraint (your notation), I can show you precisely where RScRSc​ must appear (and where it should not).

Dijiste:

Badesa et al. - 2019 - Simultaneous Scheduling of Multiple Frequency Serv.pdf

PDF

read the paper again and say read whe you finish

ChatGPT dijo:

Pensado durante un par de segundos

Read.

Badesa et al. - 2019 - Simultan…

Fuentes

Dijiste:

now, tell me why can't I just implement equation 13 when the loss of power can be smaller than the available fast frequency reserve

ChatGPT dijo:

You can implement Eq. (13) mechanically, but it stops being the “right” security condition when the contingency can be smaller than (or comparable to) the fast service you’ve scheduled—because Eq. (13) was derived under a specific assumed frequency trajectory.

1) Eq. (13) assumes the nadir happens in a particular time interval

In the paper, Eq. (13) is obtained by solving the swing equation over t∈[TS,TG)t∈[TS​,TG​) and using the expression for t\*t\* from setting RoCoF to zero. That choice is justified by the statement that the nadir will occur after TSTS​ if RS<PLRS​<PL​, which they say is the case for GB operating conditions.

Badesa et al. - 2019 - Simultan…

If your converter contingency is sometimes small, you can have:

PLconv≤RSPLconv​≤RS​

Then the net imbalance after EFR is not positive in the way assumed, and the nadir may occur before TSTS​ or you may not get a “classic” nadir in [TS,TG)[TS​,TG​) at all (frequency may start recovering earlier).

So Eq. (13) can become inapplicable to that event, not just “loose.”

2) The t\*t\* expression behind Eq. (13) changes regime

They compute:

t\*=(PL−RS)TGRGt\*=RG​(PL​−RS​)TG​​

(derived assuming the response ramps and that the system is still declining until that time).

Badesa et al. - 2019 - Simultan…

If PL−RS≤0PL​−RS​≤0, this gives t\*≤0t\*≤0, which is a tell: the assumed regime (“frequency declines until a nadir in the future”) is no longer valid. The real trajectory is governed by a different piece of the dynamics (earlier interval and/or immediate recovery).

3) Using Eq. (13) may declare security “for the wrong reason”

Because Eq. (13) contains (PL−RS)2(PL​−RS​)2, when PLPL​ is small relative to RSRS​ the RHS can become very small, and the inequality can look trivially satisfied—even though the inequality is no longer representing the true limiting condition for that event.

In other words, it may be non-binding not because the event is safe, but because the constraint is checking the wrong trajectory shape.

4) What to do instead (minimal fix)

For a “small converter loss” event, you need either:

- a piecewise nadir model that covers both regimes (nadir before TSTS​ vs after TSTS​), or

- the practical workaround: use an event-effective EFR so the model stays in the regime Eq. (13) was meant for:

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

and then apply Eq. (13) with PLconvPLconv​ and RSeff,convRSeff,conv​.

That keeps PLconv−RSeff,conv≥0PLconv​−RSeff,conv​≥0, so the timing logic used in the derivation remains consistent.

Bottom line: you can “just implement Eq. (13),” but if PLPL​ can be smaller than available fast reserve, Eq. (13)’s derivation assumptions about when the nadir occurs can fail, so it may become mathematically satisfied while physically irrelevant for that event.

Badesa et al. - 2019 - Simultan…

Fuentes

Dijiste:

t\\\*

what is this? I just see an asterisc

ChatGPT dijo:

Ah — got it 👍

You’re asking about t\*t\* (t with an asterisk).

It’s not multiplication.
It’s just a symbol used to label a specific time.

What t\*t\* means in the paper

In the paper, t\*t\* is defined as:

👉 the time when the frequency nadir occurs

In other words:

- After a disturbance (loss of generation), frequency starts dropping

- At some point it reaches its lowest value

- That moment in time is t\*t\*

So:

t\*=time at which frequency reaches its minimumt\*=time at which frequency reaches its minimum

They explicitly define it in the nomenclature as:

t\*t\*: time when the frequency nadir is reached (s)

Badesa et al. - 2019 - Simultan…

Why they need it

To enforce a nadir limit, they must evaluate:

Δf(t\*)Δf(t\*)

That is: frequency deviation at the lowest point.

To compute that, they:

- Solve the swing equation

- Find when the derivative becomes zero (minimum point)

- That time is t\*t\*

How they calculate it

They derive:

t\*=(PL−RS) TGRGt\*=RG​(PL​−RS​)TG​​

This comes from setting the rate of change of frequency to zero.

Badesa et al. - 2019 - Simultan…

Intuition

Think of frequency like a falling ball that rebounds:

frequency
   |
   |\
   | \
   |  \      ← falling
   |   \
   |    \____ ← lowest point = nadir
   |         /
   |        /
   +---------------- time
              ↑
             t*

Why this matters for your earlier question

If:

PL−RS≤0PL​−RS​≤0

then their formula gives:

t\*≤0t\*≤0

which means:

👉 the assumed “frequency keeps falling until later” behaviour is not valid

That’s exactly why Eq. (13) becomes questionable for small disturbances.

Summary

t\*t\* is simply:

✅ a label
✅ the time when frequency is lowest after a disturbance
✅ found by setting RoCoF = 0
✅ used to compute the nadir constraint

Not multiplication — just a named time point.

If you want, I can now explain graphically how the nadir moves when fast response increases.

Fuentes

Dijiste:

t\\\*\=RG​(PL​−RS​)TG​​

check how you wrote this equation, it appears with some kind of coding

ChatGPT dijo:

Ahhh yes — thanks for pointing that out 👍
That weird block looked like broken code formatting. Let me write it cleanly.

Correct expression for t\*t\*

t\*  =  (PL−RS) TGRGt\*=RG​(PL​−RS​)TG​​

Meaning of each term

- t\*t\* → time when frequency nadir occurs

- PLPL​ → size of the power loss (contingency)

- RSRS​ → fast frequency response (EFR)

- RGRG​ → primary frequency response (PFR)

- TGTG​ → delivery time of PFR

Interpretation

This tells you how long the system keeps losing frequency before it bottoms out.

- Larger loss → nadir happens later

- More fast response → nadir happens sooner

- More PFR → nadir happens sooner

If you want, I can now derive this step-by-step from the swing equation exactly as the paper does.

Dijiste:

In other words, it may be **non-binding not because the event is safe**, but because the constraint is checking the wrong trajectory shape.

but if I have another constraint considering the PL from generators where I actually know that PL>=Rs, this constraint will be binding, and it wouldn't mind that (PL-RS) is negative for the loss of converter, right?

ChatGPT dijo:

Yes — you are thinking about it correctly, and this is an important modelling insight.

Let’s go carefully and separate mathematical feasibility from physical correctness.

✅ What you are saying (and it is true)

You have:

- Generator contingency constraint

  - Uses PLgenPLgen​

  - You know PLgen≥RSPLgen​≥RS​

  - So Eq. (13) is valid in its intended regime

  - This constraint will be binding (or potentially binding)

- Converter contingency constraint

  - Uses PLconvPLconv​

  - Sometimes PLconv<RSPLconv​<RS​

  - So PLconv−RS<0PLconv​−RS​<0

You are asking:

👉 If the generator constraint already enforces enough response,
does it matter that the converter constraint is “physically wrong” but non-binding?

Answer: mathematically — no problem.

If the generator constraint guarantees security for a larger event, then:

secure for PLgen  ⇒  secure for smaller PLconvsecure for PLgen​⇒secure for smaller PLconv​

So the converter constraint being slack does not break feasibility.

✅ From an optimisation perspective

Your model will behave like:

frequency security=max⁡(generator constraint, converter constraint)frequency security=max(generator constraint, converter constraint)

If generator loss is larger:

- generator constraint binds

- converter constraint stays slack

- optimisation result unchanged

So yes:

👉 the model “does not care” if (PLconv−RS)(PLconv​−RS​) is negative
👉 because that constraint is not active anyway

⚠️ The real issue is NOT feasibility — it is interpretation

The only reason people warn about using Eq. (13) when PL<RSPL​<RS​ is:

👉 Eq. (13) was derived assuming a specific dynamic regime
👉 so if it becomes slack, it is slack for the wrong physical reason

But if another valid constraint is already stricter, then:

✔ system is still secure
✔ optimisation result still correct
✔ no mathematical inconsistency

So nothing breaks.

🧠 When this would become a problem

It becomes a problem only if:

Case A — generator constraint is NOT always larger

Example:

- sometimes converter loss is the largest contingency

- then converter constraint becomes binding

- but Eq. (13) is not valid for that regime

- then you may enforce the wrong security condition

Case B — you want event-specific analysis

If you want to correctly simulate:

- frequency trajectory for converter loss

- response procurement for converter events specifically

- pricing of services per contingency

Then physical correctness matters.

✅ Your situation (based on what you described)

If:

✔ generator loss is always ≥ converter loss
✔ same inertia and response apply
✔ you only need global frequency security

Then you are safe.

You can include both constraints and simply let:

- generator constraint bind

- converter constraint be redundant

No modelling inconsistency.

🔎 What the paper implicitly assumes

The paper models one largest infeed and schedules services against it.

Badesa et al. - 2019 - Simultan…

Everything smaller is automatically secure.

You are basically reproducing that logic but with two candidates.

🎯 Clean theoretical statement

Your formulation is valid if:

PLgen≥PLconvfor all relevant system statesPLgen​≥PLconv​for all relevant system states

because then:

security(PLgen)⊇security(PLconv)security(PLgen​)⊇security(PLconv​)

So enforcing generator security dominates.

⭐ Final answer

Yes — you are right.

If the generator-loss constraint (with PLgen≥RSPLgen​≥RS​) is binding and guarantees frequency security, then:

👉 it does not matter that (PLconv−RS)(PLconv​−RS​) is negative in the converter constraint
👉 that constraint simply becomes redundant
👉 optimisation remains correct

The only time you must fix the converter formulation is when it can become the binding contingency.

If you want, I can now show you the exact mathematical condition that guarantees one contingency dominates another in the nadir constraint (it’s not only about PLPL​; inertia and response timing also matter).

Fuentes

Dijiste:

### Case A — generator constraint is NOT always larger Example: * sometimes converter loss is the largest contingency * then converter constraint becomes binding * but Eq. (13) is not valid for that regime * then you may enforce the wrong security condition

if this is the case, would it mean that PL-RS is positive for the loss of a converter?

ChatGPT dijo:

No — not necessarily.

Even in the case where the converter contingency is the one that matters most, it does not automatically imply that

PLconv−RS>0.PLconv​−RS​>0.

Those are two different ideas:

1) “Converter constraint becomes binding”

This just means: among all your frequency constraints, the converter-loss set is the tightest one for the current operating point.

That can happen because of:

- a larger PLconvPLconv​,

- lower inertia HH in the periods when that converter is online / at risk,

- less available RGRG​ or different delivery characteristics,

- or event-specific availability (e.g., some services unavailable after that converter trips).

So “binding” ≠ “bigger PLPL​” only.

2) Sign of PLconv−RSPLconv​−RS​

This is purely:

- contingency size vs scheduled fast response capacity.

You can easily have a situation where:

- PLconvPLconv​ is the largest relevant contingency, but still smaller than your scheduled RSRS​, so

PLconv<RS⇒PLconv−RS<0.PLconv​<RS​⇒PLconv​−RS​<0.

Example:

- PLgen=800PLgen​=800 MW (smaller generator fleet online)

- PLconv=600PLconv​=600 MW (largest converter)

- you schedule RS=700RS​=700 MW because inertia is very low (high RES)

Here the converter contingency can be the critical one, yet PLconv−RS=−100PLconv​−RS​=−100 MW.

Why people worry about Eq. (13)

Eq. (13) was derived in a regime that (in their GB discussion) assumes RS<PLRS​<PL​ so the nadir happens after TSTS​.

Badesa et al. - 2019 - Simultan…

If the converter constraint is the one you rely on (it binds) and PLconv<RSPLconv​<RS​, then you’re in a different dynamic regime, so Eq. (13) may not represent the true nadir condition for that event.

When does PLconv−RSPLconv​−RS​ have to be positive?

Only if you impose it (explicitly or implicitly), e.g. by defining event-effective fast response:

RSeff,conv=min⁡(RS,PLconv)RSeff,conv​=min(RS​,PLconv​)

Then automatically:

PLconv−RSeff,conv≥0.PLconv​−RSeff,conv​≥0.

That’s why that “min” trick is useful: it forces the converter-loss constraint to stay in the regime the equation expects.

Bottom line

- Converter contingency being the binding one does not imply PLconv−RS>0PLconv​−RS​>0.

- If you need Eq. (13) to be physically consistent for converter loss, you should ensure the converter-loss constraints use an effective RSRS​ such that the residual imbalance is nonnegative.

If you tell me whether you’re modelling RSRS​ as (a) headroom/activation limit or (b) a fixed scheduled injection regardless of event size, I’ll tell you whether you actually need the min⁡min construction or not.

Fuentes

Dijiste:

how can it affect the results of my optimization problem?

ChatGPT dijo:

It can affect your optimization in two very different ways, depending on whether the converter-loss constraint ever becomes binding (or nearly binding).

1) If the generator-loss constraint always dominates

If PLgenPLgen​ is always the worst-case and your generator-loss constraints are always tighter, then the converter-loss constraint is slack.

In that case:

- Objective value: essentially unchanged

- Dispatch/commitment: unchanged

- Procured inertia/FR: unchanged

The converter constraint is just “extra paperwork” in the MILP.

So no effect, except maybe a tiny solve-time increase.

2) If the converter-loss constraint can bind (even occasionally)

Then using Eq. (13) in the regime PLconv<RSPLconv​<RS​ can distort the solution. The distortion can go either direction depending on how you modeled RSRS​.

A) Most common distortion: you may procure too little slow response/inertia (false security)

Because Eq. (13) was derived assuming a nadir in [TS,TG)[TS​,TG​), when PLconv−RS<0PLconv​−RS​<0 the formula for t\*t\* implied by the derivation no longer makes physical sense (it would suggest t\*≤0t\*≤0). The inequality can become “easy” to satisfy for the wrong reason (RHS shrinks via (PL−RS)2(PL​−RS​)2).

Optimization impact (if converter constraint binds):

- The solver may conclude “converter loss is safe” without requiring:

  - enough inertia HH, or

  - enough PFR RGRG​,

  - because EFR is “overpowering” the loss in the algebra.

This can lead to:

- fewer synchronous units committed (lower inertia),

- lower scheduled PFR,

- more RES accommodated in the model,

- lower cost solution.

But the real system might have different limiting behavior (nadir earlier than assumed, overshoot constraints, control saturation, deadbands, etc.). So you could be optimizing against a constraint that isn’t the right physics for that event.

B) Less common: you may procure too much (overly conservative) if you add ad-hoc fixes incorrectly

If instead you impose something like PLconv≥RSPLconv​≥RS​ (or force response to behave like the larger event), you might:

- prevent EFR from being used efficiently,

- keep more thermal units online,

- increase curtailment,

- raise operating cost.

3) A concrete “what changes?” checklist

When the converter-loss constraint becomes binding and you model it incorrectly (using Eq. 13 with PLconv<RSPLconv​<RS​ and no regime handling), it can change:

- Committed synchronous generation
↓ if the model thinks EFR alone handles it → inertia requirement may not bite.

- Scheduled PFR RGRG​
↓ because Eq. (13) may stop demanding it in that regime.

- BESS operation / EFR scheduling
Potentially ↑ RSRS​ because it looks like a cheap way to “solve” constraints (depending on how you price/limit it).

- RES curtailment & emissions
↓ curtailment, ↓ emissions in the optimization, but possibly unrealistic if security is misrepresented.

- Marginal prices / shadow values of services
Can become distorted: EFR may look less/more valuable than it truly is.

4) The safe modeling tweak and how it affects results

If you replace RSRS​ by an event-effective:

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

then for the converter-loss constraints you avoid the “over-response” regime.

Optimization impact:

- Converter-loss constraints become slightly tighter whenever RS>PLconvRS​>PLconv​, because extra EFR beyond the event doesn’t help the inequality.

- That can increase:

  - required inertia HH and/or PFR RGRG​ in those hours,

  - commitment of synchronous units,

  - operating cost (sometimes modest, sometimes meaningful if low-inertia hours are frequent).

But it makes the constraint represent the intended regime, so your solution is less likely to be “cheap but based on the wrong physics.”

5) Quick rule of thumb

- If converter-loss constraints are never binding → almost no effect.

- If they sometimes bind and you allow PLconv<RSPLconv​<RS​ in Eq. (13) without regime handling → you risk incorrectly low inertia/PFR and an optimistically low cost.

- Using RSeff,convRSeff,conv​ (or a piecewise nadir constraint) → potentially higher cost, but more physically consistent.

If you tell me two things, I can be very specific about the direction and size of the effect in your setup:

- Is RSRS​ free/cheap in the objective (like in the paper’s “provision is free” framing) or does it have an explicit cost?

Badesa et al. - 2019 - Simultan…

- Can the converter contingency ever be larger than the generator contingency in your dataset/operating states?

Fuentes

Dijiste:

, when PLconv−RS<0P\_L^{conv} - R\_S < 0PLconv​−RS​<0

but in this case, wouldn't the generator loss be the binding constraint?

ChatGPT dijo:

Yes — in the situation you are describing, most likely yes.
If the generator loss is larger and all other conditions are the same, then the generator-loss constraint will be the binding one.

But let’s state this precisely and rigorously.

✅ If these three things are true

1️⃣ Generator loss is always larger:

PLgen≥PLconvPLgen​≥PLconv​

2️⃣ Same resources respond to both events:

- same inertia HH

- same EFR RSRS​

- same PFR RGRG​

- same delivery times

3️⃣ Same frequency limits apply

✅ Then generator loss dominates mathematically

All frequency constraints (RoCoF, nadir, steady-state) become harder to satisfy when the disturbance is larger, because:

- RoCoF ∝ disturbance

- steady-state imbalance ∝ disturbance − response

- nadir severity increases with disturbance (see Eq. 13 structure)

Badesa et al. - 2019 - Simultan…

So if the system is secure for a larger loss, it is automatically secure for a smaller one.

This means:

feasible region for generator loss⊆feasible region for converter lossfeasible region for generator loss⊆feasible region for converter loss

So the generator constraint binds, converter constraint is redundant.

✅ What happens if PLconv−RS<0PLconv​−RS​<0

Nothing operationally important if generator constraint binds.

The converter constraint just sits there satisfied.

The optimizer ignores it because a stricter constraint already exists.

This is completely normal in optimization models.

⚠️ When this would NOT be true

Generator loss will not necessarily bind if any of these differ by contingency:

- inertia after event

- response availability

- response speed

- different control behaviour

- location-dependent frequency effects

- different post-fault topology

Then a smaller disturbance could still be harder to manage.

But if you are using the uniform frequency model of the paper, with same parameters, that does not happen.

Badesa et al. - 2019 - Simultan…

🎯 Direct answer to your question

If PLconv−RS<0PLconv​−RS​<0, wouldn't generator loss be the binding constraint?

Yes — if generator loss is larger and system parameters are the same.

Then:

✔ generator loss constraint binds
✔ converter constraint is redundant
✔ negative PLconv−RSPLconv​−RS​ does not matter
✔ optimization results unchanged

🧠 Conceptual interpretation

You are simply enforcing security against:

👉 worst credible contingency

Everything smaller is automatically covered.

That is exactly how system security standards work.

⭐ Final takeaway

Your logic is correct if generator loss is always the largest credible contingency in the model.

In that case:

👉 you do NOT need to modify Eq. (13) for the converter
👉 you do NOT need effective EFR
👉 converter constraint is informational only

You are just checking an easier case.

If you want, I can now give you the precise mathematical proof that Eq. (13) is monotonic in PLPL​ under fixed H,RS,RGH,RS​,RG​. That formally proves dominance.

Fuentes

Dijiste:

what happens if the inertia for the converters and for generators is different?

ChatGPT dijo:

If the inertia “seen” by the system depends on which infeed trips, then a smaller converter contingency can absolutely become more binding than a larger generator contingency. In that case, you can’t assume “largest PLPL​ dominates” anymore.

Why inertia can differ by contingency

After a trip, the inertia available to arrest frequency is the inertia of the machines (and any synthetic inertia) that remain electrically coupled to the frequency that matters. A converter trip can change that in ways a generator trip doesn’t, e.g.:

- HVDC/link trip can split the system or reduce synchronizing strength → part of the synchronous fleet may no longer contribute to the local frequency.

- Converter-connected resources might be providing synthetic inertia / fast frequency response that is lost with the converter.

- Different contingencies can create different post-fault topologies or “islanding,” changing the effective inertia.

The paper uses a single uniform-frequency inertia HH (their eq. (8)) and applies it to the single largest loss PLPL​.

Badesa et al. - 2019 - Simultan…

Once HH becomes contingency-dependent, you’re outside that simplification.

What this does to the constraints

RoCoF is the clearest:

∣RoCoF∣=PLf02H∣RoCoF∣=2HPL​f0​​

So the “stress” is essentially PL/HPL​/H. A smaller loss with much smaller inertia can be worse.

Example (numbers just to show the point):

- Generator loss: PLgen=1800PLgen​=1800 MW, Hgen=200Hgen=200 GWs
⇒PL/H=9⇒PL​/H=9

- Converter loss: PLconv=900PLconv​=900 MW, Hconv=50Hconv=50 GWs
⇒PL/H=18⇒PL​/H=18

Even though PLconvPLconv​ is half, RoCoF is twice as bad → converter constraint binds.

Nadir and steady-state can flip too

The nadir inequality (eq. 13 / 19 form) scales with HH in the stabilizing term Hf0f0​H​ and with the disturbance terms involving PLPL​. Lower HH makes it harder to satisfy the nadir requirement.

Badesa et al. - 2019 - Simultan…

So your “dominance” logic changes from:

“largest PLPL​ dominates”

to:

“largest effective severity dominates,” roughly driven by PL/HPL​/H (RoCoF) and by the full nadir expression.

Optimization impact

If HH differs by contingency, then adding a converter-loss constraint can change your solution materially:

- More synchronous commitment in hours where converter-loss inertia is low

- More PFR/EFR procurement (or different mix)

- Potentially deloading or limiting converter infeed (if you model that as a decision)

- Higher operating cost / different emissions / different curtailment

And crucially, the converter constraint can bind even if PLconv<PLgenPLconv​<PLgen​.

How to model it cleanly

1) Introduce contingency-specific inertia

Use:

- HgenHgen in the generator-loss constraints

- HconvHconv in the converter-loss constraints

Conceptually:

Hk=∑i∈onlineHikHk=i∈online∑​Hik​

where the set contributing inertia may differ by contingency kk.

If the only difference is “some resources are not available after this outage,” you can model:

- different subtraction terms (like their “lost infeed inertia” concept in eq. (8)), but contingency-specific.

Badesa et al. - 2019 - Simultan…

2) Then enforce constraints for both contingencies

RoCoF(PLgen,Hgen)(PLgen​,Hgen), Nadir(PLgen,Hgen,… )(PLgen​,Hgen,…)
and
RoCoF(PLconv,Hconv)(PLconv​,Hconv), Nadir(PLconv,Hconv,… )(PLconv​,Hconv,…)

Now whichever is tighter will bind.

3) Revisit the PL<RSPL​<RS​ regime if converter constraints can bind

If converter-loss constraints can bind and you can have PLconv<RSPLconv​<RS​, then you should use the regime-handling we discussed (e.g. RSeff,conv=min⁡(RS,PLconv)RSeff,conv​=min(RS​,PLconv​)) or a piecewise nadir constraint—otherwise Eq. (13)’s derivation assumptions can misrepresent the converter event.

Bottom line

When inertia differs by contingency, the binding event is not necessarily the one with the largest PLPL​. A smaller converter loss can be the critical one if it “sees” much lower inertia (or loses synthetic inertia/fast response with it). In that case, adding a converter-specific PLPL​ and using the same constraint structure does make sense—but you should treat HH (and possibly available RS,RGRS​,RG​) as contingency-specific too.

Badesa et al. - 2019 - Simultan…

Fuentes

Dijiste:

but I know that the loss of a converter doesn't see a lower inertia

ChatGPT dijo:

If you’re confident that a converter loss does not change the effective inertia (same HH “seen” by the system as for a generator loss), then you’re back in the simpler regime:

- same HH,

- same RS,RGRS​,RG​ availability and delivery assumptions,

- same frequency limits,

and the only thing that differs is the disturbance size PLPL​.

What that implies

In that case, a smaller converter loss is (almost always) easier to satisfy than the larger generator loss. So:

- the generator-loss constraints dominate,

- the converter-loss constraints are redundant (slack),

- your optimization result won’t change by adding the converter set.

This aligns with the paper’s basic “schedule for the largest infeed” logic.

Badesa et al. - 2019 - Simultan…

So why worry about PLconv−RS<0PLconv​−RS​<0?

If the converter-loss constraints are never binding, then practically you don’t need to worry: the solver will ignore them.

The only remaining reasons to care are:

- Numerics/solver time: extra binaries/constraints can slow the MILP a bit.

- Interpretability / post-analysis: if you later look at “marginal values” or “which constraint is active,” the converter constraint might look weird (because it lives in a regime the derivation didn’t target), even if it’s slack.

One subtle edge case

If you have any other contingency-dependent differences, even if inertia is the same—e.g.

- converter loss has different response activation (some EFR/PFR unavailable),

- different deadband/delay assumptions,

- different “largest infeed” logic (e.g., converter loss is treated as credible at times generator isn’t),
then it could bind.

But under your stated assumption (“converter loss doesn’t see lower inertia” and otherwise same services), the safe conclusion is:

✅ Yes: the generator loss will be the binding security driver; the converter-loss Eq. (13) can be left in, but it won’t affect the optimum.

Practical recommendation

If your goal is just to ensure security, simplest is:

PL=max⁡(PLgen, PLconv)PL​=max(PLgen​,PLconv​)

and keep a single set of constraints.

If your goal is reporting/assessment by contingency, you can keep both sets, but don’t expect the converter set to change results.

If you want, tell me how you define PLconvPLconv​ (fixed rating? time-varying flow limit?) and I’ll suggest the cleanest way to integrate it without adding unnecessary binaries.

Fuentes

Dijiste:

**B2) Piecewise “min” (more accurate, MILP-able):** RSeff,conv\=min⁡(RS, PLconv)R\_S^{\\text{eff,conv}} = \\min(R\_S,\\, P\_L^{\\text{conv}})RSeff,conv​\=min(RS​,PLconv​) This is easy to linearize with standard MILP constructs (additional binary + big-M), and it makes the constraints behave correctly when PLconv<RSP\_L^{\\text{conv}}<R\_SPLconv​<RS​. Then in the converter-loss version of (13)/(19)/(9) you replace RSR\_SRS​ by RSeff,convR\_S^{\\text{eff,conv}}RSeff,conv​ and also replace (PL−RS)(P\_L-R\_S)(PL​−RS​) by (PLconv−RSeff,conv)(P\_L^{\\text{conv}}-R\_S^{\\text{eff,conv}})(PLconv​−RSeff,conv​). This keeps the math consistent with the assumption that the “residual imbalance after fast response” is nonnegative.

Badesa et al. - 2019 - Simultaneous Scheduling of Multiple Frequency Serv.pdf

PDF

elavorate more on this option. I provided the file again in case that you need it.

ChatGPT dijo:

Pensado durante un par de segundos

Sure — let’s elaborate the B2 “min” option carefully and tie it back to how Eq. (13) is derived in the paper.

Why the “min” shows up at all

Eq. (13) is obtained by:

- assuming the nadir happens in the interval t∈[TS,TG)t∈[TS​,TG​), which they justify by saying the nadir is after TSTS​ if RSRS​ is lower than the largest infeed (“certainly the case” in GB)

Badesa et al. - 2019 - Simultan…

- using t\*=(PL−RS)TGRGt\*=RG​(PL​−RS​)TG​​

Badesa et al. - 2019 - Simultan…

If you plug a contingency where PL<RSPL​<RS​, then:

- PL−RS<0PL​−RS​<0

- their t\*t\* becomes ≤0≤0, which is outside the time interval they solved the swing equation on

- so Eq. (13) is no longer the “right” nadir condition for that event, even if it looks slack

So the “min” trick is a way to keep the converter-loss constraint in the regime that Eq. (13) was meant to represent.

The idea

For the converter-loss event, define an effective fast response actually relevant to that event:

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

Interpretation:

- You may procure RSRS​ (capacity available),

- but for a small event you shouldn’t “credit” more fast response than the lost power when building the nadir algebra that assumed PL−RS≥0PL​−RS​≥0.

This enforces:

PLconv−RSeff,conv≥0PLconv​−RSeff,conv​≥0

so the residual imbalance term used in the paper’s derivation stays physically consistent for that contingency.

How you use it in the paper’s constraints

You build a second copy of the frequency constraints for the converter outage, but you replace:

- PL→PLconvPL​→PLconv​

- RS→RSeff,convRS​→RSeff,conv​

- and anywhere (PL−RS)(PL​−RS​) appears, use (PLconv−RSeff,conv)(PLconv​−RSeff,conv​)

Converter-loss version of Eq. (13)

From paper Eq. (13):

Badesa et al. - 2019 - Simultan…

(Hf0−RS TS4Δfmax⁡)RG≥(PL−RS)2 TG4Δfmax⁡(f0​H​−4Δfmax​RS​TS​​)RG​≥4Δfmax​(PL​−RS​)2TG​​

Converter version:

(Hf0−RSeff,conv TS4Δfmax⁡)RG≥(PLconv−RSeff,conv)2 TG4Δfmax⁡(f0​H​−4Δfmax​RSeff,conv​TS​​)RG​≥4Δfmax​(PLconv​−RSeff,conv​)2TG​​

Same structure, but now you never enter the “PL−RS<0PL​−RS​<0” regime that breaks the derivation logic.

If you’re using the damping-improved version (Eq. 19/21/23), do the same replacement there too.

Badesa et al. - 2019 - Simultan…

MILP linearization of the min

Introduce:

- RScRSc​ for RSeff,convRSeff,conv​

- binary z∈{0,1}z∈{0,1}

- tight big-M M≥max⁡(RSmax⁡,(PLconv)max⁡)M≥max(RSmax​,(PLconv​)max)

Constraints:

Upper bounds

RSc≤RSRSc​≤RS​RSc≤PLconvRSc​≤PLconv​

Lower bounds (activate the correct branch)

RSc≥RS−M(1−z)RSc​≥RS​−M(1−z)RSc≥PLconv−MzRSc​≥PLconv​−Mzz∈{0,1}z∈{0,1}

This enforces exactly RSc=min⁡(RS,PLconv)RSc​=min(RS​,PLconv​).

What this changes in your optimization results

If generator loss always dominates

Then the converter constraints are slack anyway, and this “min” won’t change anything except maybe solver time.

If converter loss can ever be the binding contingency

Then the “min” matters a lot:

- Without it, Eq. (13) can become trivially satisfied for small events for the wrong reason (because you are outside the derivation regime).

- With it, converter-loss security is enforced in a regime consistent with the paper’s assumptions, which can require:

  - more RGRG​,

  - more inertia HH,

  - or a different mix of services in those hours.

So the “min” is basically a guardrail that prevents the solver from exploiting a modelling regime the paper did not intend Eq. (13) to cover.

When I would not bother with this

If you truly know:

- PLgen≥PLconvPLgen​≥PLconv​ always,

- same HH, same RGRG​, same RSRS​ applicability,

- and you’re only enforcing “secure for worst case,”

then set

PL=max⁡(PLgen,PLconv)PL​=max(PLgen​,PLconv​)

and keep one set of constraints. That is closest to the paper’s “largest infeed” security framing (see Eq. (3) and surrounding discussion).

Badesa et al. - 2019 - Simultan…

If you tell me how you define PLconvPLconv​ (fixed rating? time-varying measured flow? seasonal cap?), I can show the cleanest way to bound MM tightly and keep the MILP numerically stable.

Fuentes

Dijiste:

so, you are enforcing the Pl from the convertor to be always larger than Rs to keep the constraints in the assumed regime, right?

ChatGPT dijo:

Not exactly.

I’m not forcing PLconvPLconv​ to be larger than RSRS​.
I’m keeping your physics constraint in the same regime by changing what you credit as fast response for that specific contingency.

What the “min” does

It defines an event-effective fast response:

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

That guarantees:

PLconv−RSeff,conv≥0PLconv​−RSeff,conv​≥0

So the residual imbalance term that appears in the paper’s derivation (and in t\*t\*) stays nonnegative, which is the regime they implicitly rely on when they solve for the nadir over t∈[TS,TG)t∈[TS​,TG​).

Badesa et al. - 2019 - Simultan…

Key difference

- You are not modifying the contingency PLconvPLconv​.

- You are not imposing PLconv≥RSPLconv​≥RS​.

- You are only saying: for a small event, it doesn’t make sense (in this analytical form) to “use” more EFR than the event size when computing the post-fault imbalance.

Intuition

If PLconv=200PLconv​=200 MW and you have RS=600RS​=600 MW available, the model would set:

RSeff,conv=200 MWRSeff,conv​=200 MW

meaning: you can’t reduce a 200 MW deficit by 600 MW in the direction assumed by the nadir derivation; the extra 400 MW doesn’t help that particular inequality—it would instead imply over-response / different trajectory shape.

If you actually wanted to force the regime

That would be a different (and usually undesirable) constraint:

PLconv≥RSPLconv​≥RS​

That would change your solution a lot (it would limit how much fast response you can schedule), and it’s not what I’m recommending.

So: we’re enforcing nonnegativity of the residual PLconv−RSeff,convPLconv​−RSeff,conv​, not making the converter loss bigger.

Fuentes

Dijiste:

Not exactly. I’m **not** forcing PLconvP\_L^{conv}PLconv​ to be larger than RSR\_SRS​. I’m keeping your _physics constraint_ in the same regime by changing what you **credit** as fast response for that _specific_ contingency. ### What the “min” does It defines an event-effective fast response: RSeff,conv\=min⁡(RS,  PLconv)R\_S^{eff,conv}=\\min(R\_S,\\;P\_L^{conv})RSeff,conv​\=min(RS​,PLconv​) That guarantees: PLconv−RSeff,conv≥0P\_L^{conv}-R\_S^{eff,conv}\\ge 0PLconv​−RSeff,conv​≥0 So the _residual imbalance term_ that appears in the paper’s derivation (and in t\\\*t^\\\*t\\\*) stays nonnegative, which is the regime they implicitly rely on when they solve for the nadir over t∈\[TS,TG)t\\in\[T\_S,T\_G)t∈\[TS​,TG​).

I don't understand this part, may you elavorate on this?

ChatGPT dijo:

Yes — absolutely 👍
Let’s slow this down and explain it step by step with intuition, because this is the core conceptual point.

You’re confused about:

👉 Why do we replace RSRS​ with

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

👉 What does “keeping the constraint in the same regime” actually mean?

🔵 Step 1 — What the paper assumes physically

The paper models what happens after a generation loss.

Immediately after the loss:

power imbalance=lost generation−fast responsepower imbalance=lost generation−fast response

They assume:

PL>RSPL​>RS​

Meaning:

👉 the loss is bigger than the fast response

So initially:

- the system still has a deficit

- frequency keeps falling

- fast response only slows the fall

Then later:

- slower PFR ramps in

- frequency reaches a minimum (nadir)

- then recovery begins

This is the trajectory they mathematically solve.

They explicitly use this when computing the time of nadir:

t∗=(PL−RS)TGRGt∗=RG​(PL​−RS​)TG​​

Badesa et al. - 2019 - Simultan…

This only makes physical sense if:

PL−RS>0PL​−RS​>0

because that means:

👉 the system keeps losing frequency for some time
👉 nadir happens later

🔵 Step 2 — What happens if the event is small

Now suppose:

- fast response = 600 MW

- converter loss = 200 MW

So:

PLconv−RS=−400PLconv​−RS​=−400

What does that mean physically?

It means:

👉 fast response is larger than the disturbance

So immediately after the event:

- response is already more than enough

- frequency may stop falling almost instantly

- or may even rise after a tiny dip

The system does NOT follow the trajectory the paper solved.

So:

❌ the nadir does not occur in the same time region
❌ their formula for t∗t∗ is not valid
❌ Eq. (13) is not the correct condition for that case

🔵 Step 3 — Why this breaks the math

The paper solved the swing equation only in this interval:

t∈[TS,TG)t∈[TS​,TG​)

They justify this by saying nadir occurs after fast response is fully delivered when RS<PLRS​<PL​.

Badesa et al. - 2019 - Simultan…

But if:

PL<RSPL​<RS​

then:

- nadir may happen earlier

- or the system may never even enter that region

So their derived inequality is checking the wrong physics.

🔵 Step 4 — What the “min” fixes

Instead of changing the event size, we change what we count as useful fast response for that event.

Define:

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

This says:

👉 for a small event, only the part of fast response up to the size of the loss is relevant for the deficit

Example:

| available fast response | event size | effective |
| --- | --- | --- |
| 600 MW | 200 MW | 200 MW |

🔵 Step 5 — What this does mathematically

Now the residual imbalance becomes:

PLconv−RSeff,convPLconv​−RSeff,conv​

Which is:

- zero if response ≥ loss

- positive if loss > response

Never negative.

So now the model always describes:

👉 a system that either still has deficit or exactly balances
👉 never an “over-response” regime

That matches the structure used in the derivation.

🔵 Step 6 — What this means physically

We are not changing the event.

We are saying:

👉 you can’t treat “extra response beyond the loss” as helping reduce the deficit in the same way the derivation assumes

Because once the loss is fully compensated:

- the dynamic behavior changes

- the frequency trajectory shape changes

- the analytical formula changes

So we cap the reminder to stay in the same physics model.

🔵 Step 7 — Analogy (very important)

Imagine braking a car.

Car speed = 20 km/h
Brake capacity = enough to stop from 100 km/h

Do you compute stopping distance using full brake capacity as if you were at 100 km/h?

No.

You only consider what is needed to stop from 20 km/h.

Extra braking power doesn’t make the same formula valid.

🔵 Step 8 — What “same regime” really means

It means:

👉 same qualitative behavior of frequency trajectory

Specifically:

- frequency declines after event

- fast response alone does not stop decline

- nadir occurs after fast response is fully delivered

That is the regime the paper solved.

🔵 Step 9 — Why we do NOT change PLconvPLconv​

Because the event size is physical.

We are not saying the converter loss is bigger.

We are saying:

👉 the analytical model only applies to the portion of response that acts against a remaining deficit

🔵 Step 10 — When you actually don’t need this

If generator loss is always larger and binding, then:

✔ converter constraint never binds
✔ trajectory regime irrelevant
✔ no need for min

That’s why earlier I said it may not affect your optimization.

⭐ Final simple explanation

The paper assumes:

👉 loss is bigger than fast response
👉 frequency keeps falling before nadir

If a converter loss is smaller than fast response, the system behaves differently.

So we cap the “usable fast response” at the event size so the math still describes the same kind of frequency trajectory.

We are not making the loss bigger.

We are preventing the model from entering a different physical regime that the equation was not derived for.

If you want, next I can draw the two different frequency trajectories (loss > response vs response > loss). That makes everything instantly clear.

Fuentes

Dijiste:

so, does it mean that we are enforcing the nadir constraints to use just a part of the reserve to hold the frequency stability constraint when a converter fails, and this loss of power is smaller than Rs, but if the loss of power is larger than Rs, then the values of the auxiliary variables are equal to the values of the Rs?

ChatGPT dijo:

Yes — that is exactly the correct interpretation.
You’ve got it 👍

Let me restate it cleanly and precisely so there is zero ambiguity.

🎯 What the auxiliary variable represents

You define an auxiliary variable (for the converter contingency):

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

This is the portion of fast reserve that is actually relevant for the converter-loss nadir calculation.

It does not change what you procure.
It only changes what the nadir constraint “sees”.

🎯 What happens in each case

✅ Case 1 — converter loss is smaller than fast reserve

PLconv<RSPLconv​<RS​

Then:

RSeff,conv=PLconvRSeff,conv​=PLconv​

Interpretation:

✔ Only part of the available fast reserve is needed to balance the loss
✔ The nadir constraint uses only that part
✔ The remaining reserve is irrelevant for that specific disturbance

So yes — exactly as you said:

👉 the nadir constraint uses only a portion of the reserve

✅ Case 2 — converter loss is larger than fast reserve

PLconv≥RSPLconv​≥RS​

Then:

RSeff,conv=RSRSeff,conv​=RS​

Interpretation:

✔ All available fast reserve is used
✔ The nadir constraint sees the full scheduled fast reserve

So again — exactly what you said:

👉 the auxiliary variable equals the full reserve

🎯 Why this is done (conceptual meaning)

You are modeling:

👉 how much fast response is actually acting against the remaining deficit after the disturbance

If the loss is small, the full reserve is not acting against a deficit (because the deficit is already fully covered).

If the loss is large, all reserve is relevant.

So the auxiliary variable represents:

⭐ effective fast response that contributes to deficit reduction for that specific contingency

🎯 How this appears in the nadir equation

Original nadir condition (paper Eq. 13):

Badesa et al. - 2019 - Simultan…

(Hf0−RSTS4Δfmax)RG≥(PL−RS)2TG4Δfmax(f0​H​−4Δfmax​RS​TS​​)RG​≥4Δfmax​(PL​−RS​)2TG​​

Converter-loss version:

(Hf0−RSeff,convTS4Δfmax)RG≥(PLconv−RSeff,conv)2TG4Δfmax(f0​H​−4Δfmax​RSeff,conv​TS​​)RG​≥4Δfmax​(PLconv​−RSeff,conv​)2TG​​

So the equation automatically switches behavior depending on event size.

🎯 Physical interpretation (very important)

You are not dispatching reserve differently.

You are just saying:

👉 only the portion of reserve that offsets a real deficit matters for the nadir mechanism modeled in the paper

Extra reserve beyond that changes the trajectory type, which the equation was not derived for.

🎯 What your statement means in precise optimization language

Yes — you are enforcing that:

- the nadir constraint uses only the effective part of reserve that participates in balancing the disturbance,

- and this effective part equals:

  - the loss size if the loss is smaller than reserve,

  - the full reserve otherwise.

That is mathematically identical to:

RSeff,conv=min⁡(RS,  PLconv)RSeff,conv​=min(RS​,PLconv​)

⭐ Final confirmation

Your sentence:

we are enforcing the nadir constraints to use just a part of the reserve when the converter loss is smaller than Rs, but if the loss is larger than Rs, then the auxiliary variable equals Rs

✅ Completely correct.

That is precisely the modeling interpretation.

If you want, next I can explain one subtle but important consequence:

👉 how this affects marginal prices / shadow values of fast reserve in the optimization.

Fuentes

Top of Form

Bottom of Form